GreatPower Lithium AA Battery 3500mAh 1.5V

Battery Type

Lithium (non-rechargeable)

Product Information

Model: FR6 AA Lithium Battery

Nominal Voltage: 1.5V

Capacity: 3500mAh

Chemical System: Lithium and Iron Disulfide Battery（Li/FeS2）

| Ingredient | Percent | CAS Index No./EC No. | Molecular formula |
| --- | --- | --- | --- |
| Iron Sulfide Compound | 30.6% | 12068-85-8 | FeSx |
| Lithium | 6.4% | 7439-93-2 | Li |
| Organic Solvent | 14.8% | 646-06-0 |  |
| Lithium Salt | 1.9% | 7791-03-9 |  |
| Polypropylene | 2.5% | 25085-53-4 |  |
| Steel | 35.7% | 7439-89-6 | Fe |
| Aluminum | 8.1% | 7429-90-5 | Al |

Replaceability

These batteries are replaceable and are not permanently built into devices. They are designed to be removed and replaced by the user. However, some devices (e.g. toys or medical devices) may require a screwdriver or a similar tool for access.

Performance Durability

The storage life of AA lithium batteries is 10 years. The warranty period is 1 year.

Recyclability Information

Lithium batteries must not be disposed of with household waste.

They must be delivered to authorized recycling centers or collection points, especially because they may pose fire risk.

Please check with your local recycling center for proper disposal guidance.

Best before

This is printed on the product and/or product packaging. Please refer to the label for the best before date.

Safety Warnings

EN Do not recharge or put in fire.

DE Nicht versuchen aufzuladen oder ins Feuer zu legen.

FR Ne tentez pas de recharger ou d'exposer au feu.

IT Non tentare di ricaricare o di esporre al fuoco.

ES Nointente recargar ni exponer al fuego.

NL Probeerniet op te laden of bloot te stellen aan vuur.

PT Naorecarregue nem coloque no fogo.

JP 充電したり、火の中に投げ込んだりしないでください。

BE Niet opladen of in het vuur gooien.

SV Nie ładować ani wrzucać do ognia.

PL Nie tadowac ani wrzucac do ognia.

Flash Point: N/A.

Auto-Ignition Temperature: N/A.

Extinguishing Media: Water, CO2.

Special Fire-Fighting Procedures

Self-contained breathing apparatus.

Unusual Fire and Explosion Hazards

Cell may vent when subjected to excessive heat-exposing battery contents.

Hazardous Combustion Products

Carbon monoxide, carbon dioxide, lithium oxide fumes.

Intended Use

Non-rechargeable lithium batteries are designed for use in high-performance and long-lasting applications, such as:

- Cameras and camera flashes

- Smoke detectors

- Remote sensors

- Medical devices (non-critical)

- Security systems

- Calculators

- Handheld instruments

- Toys

They are not intended for recharging. Attempting to recharge these batteries can be dangerous.

Manufacturer Information

Brand: GreatPower

Manufacturer: Guangzhou Great Power Energy & Techology Co.,Ltd

Address: No. 912 Shiliang Road, Shawan town, Panyu, Guangzhou, China